# ReputeTap Physical Standee Specifications & Fabricator Brief

## 1. Physical Specifications

- Material: 3mm to 4mm High-Gloss Cast Clear Acrylic (A-grade).

- Format: L-Shape or T-Shape Counter Desk Standee.

- Dimensions:

- Width: 100 mm (4 inches)

- Height: 150 mm (6 inches)

- Base: 50 mm slanted return (for 75° viewing angle at cash counters)

- Print Method: Direct UV Flatbed Printing (Reverse-printed on back of clear acrylic or front-printed on white acrylic core with scratch-resistant matte varnish).

- Colors:

- Background: Crisp White (#FFFFFF) or Matte Black (#111827) depending on venue aesthetic.

- Accent colors: Official Google review star gold (#FBBC05) and Google Blue (#4285F4).

## 2. Component Layout & Hierarchy

+------------------------------------------------+

|                   [Client Logo]                |

|               (Max height: 28mm)               |

|                                                |

|             "How Was Your Visit?"              |

|              ★★★★★ (Gold Foil/Print)           |

|                                                |

|                   ((  NFC  ))                  |

|                 TAP PHONE HERE                 |

|             (NFC chip placed behind)           |

|                                                |

|                     [ QR CODE ]                |

|                    (35mm x 35mm)               |

|                  "Or Scan to Rate"             |

|                                                |

|        Google Partner / Verified Review Mark   |

|         "Speak with Manager Option Inside"     |

+------------------------------------------------+

## 3. NFC Chip Placement

- Chip Type: NTAG213 (144 bytes user memory) or NTAG215 (504 bytes).

- Placement: Affixed to the backside of the acrylic standee directly behind the "TAP PHONE HERE" icon using circular 25mm adhesive NFC sticker or embedded during acrylic sandwich lamination.

- Ferrite Anti-Metal Layer: If the standee is placed on a metal cash drawer / POS terminal, use an "On-Metal" anti-ferrite shielded NFC tag so the signal is not absorbed by the metal counter.

## 4. Bill of Materials (BOM) & Unit Economics (Per Standee)

- Laser-cut 3mm Acrylic Standee: ₹120 – ₹180 ($1.45 – $2.15)

- UV Double-Pass Print: ₹60 – ₹90 ($0.70 – $1.10)

- NTAG213 25mm NFC Tag: ₹15 – ₹25 ($0.18 – $0.30)

- Packaging & Protective Sleeve: ₹15 ($0.18)

- Total Unit Cost: ₹210 – ₹310 (~$2.50 – $3.70)

- Selling Price (Pair of 2 Standees + Digital Setup): ₹2,999 (~$36.00)

- Gross Profit Margin: 80% to 85%